ILC Damping Ring Optics data file “ILC-eDR-Optics.txt”:

Each line corresponds to one component, ordered from the origin of the beam line.

Column 1-5 (AX      BX      NX      EX     EPX): horizontal alpha, beta, phase advance, dispersion, angular dispersion, at the entrance of the component.

Column 10-14 (AY      BY      NY      EY     EPY): Same for vertical.

Column 6 (Element): name of the component.

Column 7 (Length): length of the component.

Column 8 (Value): value of the default-keyword of the component (depends on the component type)

Column 9 (s(m)): distance from the origin to the entrance of the component (along the beam line).

Column 15 (DetR): determinant of the coupling matrix R at the entrance of the component. (all zero in our case)

Column 16 (#): component number from the origin.