|  | Development of a Low-Latency, High-Precision, Intra-Train Beam Feedback System Based on Cavity Beam Position Monitors |
| --- | --- |

A low-latency, intra-train, beam feedback system utilising a cavity beam position monitor (BPM) has been developed and tested at the final focus of the Accelerator Test Facility (ATF2) at KEK. A low-Q cavity BPM was utilised with custom signal processing electronics, designed for low latency and optimal position resolution, to provide an input beam position signal with a resolution of approximately 100nm. A custom stripline kicker and power amplifier, and an FPGA-based digital feedback board, were used to provide beam correction and feedback control, respectively. The system was deployed in single-pass, multi-bunch mode with the aim of demonstrating intra-train beam stabilisation on electron bunches of charge ~ 1nC separated in time by c. 280ns. The feedback latency was measured to be c. 160ns and the beam position was stabilised to c. 100nm.

A Sub-Micron Resolution, Wide-Band, Stripline Beam Position Monitoring System and its Application in Beam Stabilisation

A low-latency, sub-micron resolution stripline beam position monitoring (BPM) system has been developed and tested with beam at the KEK Accelerator Test Facility (ATF2), where it has been used as part of a beam stabilisation system. The fast analogue front-end signal processor is based on a single-stage RF down-mixer and a position resolution as low as 400 nm has been demonstrated for beam intensities of ~1 nC, with single-pass beam. The BPM position data are digitised by fast ADCs on an FPGA-based digital feedback controller, which is used to drive either a pair of kickers local to the BPMs and nominally orthogonal in phase, in closed-loop feedback mode, or a downstream kicker in the ATF2 final focus region, in feedforward mode. The beam jitter is measured downstream of the final focus system with high resolution, low-Q, cavity BPMs, and the relative performance of both systems in stabilising the beam is compared.

The Applications of Stripline and Cavity Beam Position Monitors in Low-Latency, High-Precision, Intra-Train Feedback Systems

Two, low-latency, sub-micron beam position monitoring (BPM) systems have been developed and tested with beam at the KEK Accelerator Test Facility (ATF2). One system, based on stripline BPMs uses fast analogue front-end signal processing and has demonstrated a position resolution as low as 400nm for beam intensities of ~1 nC, with single-pass beam. The other, based on low-Q cavity BPMs and utilising custom signal processing electronics designed for low latency provides a single pass resolution of approximately 100nm. The BPM position data are digitised by fast ADCs on an FPGA-based digital feedback controller and used in three modes: the upstream BPM data driving a pair of upstream kickers nominally orthogonal in phase in closed-loop feedback mode; the upstream BPM data driving a downstream kicker in the ATF2 final focus region in feedforward mode and the cavity BPM data driving a downstream kicker in the ATF2 final focus region. The beam jitter is measured downstream of the final focus system with the cavity BPMs. The relative performance of both systems in stabilising the beam is compared.