| Parameter | Value | Comments |
| --- | --- | --- |
| Experiment parameters |  |  |
| Experiment lifetime | 15y | Without performance degradation |
| Total integrated luminosity |  |  |
| Bunch structure and timing |  |  |
| Events/bunch crossing |  |  |
| Beam spot variation | Longitudinal: Radial: | Radially this includes the variations in beam orbit and the precision with which the tracker can be positioned with respect to the beam orbit |
| Magnetic field | 5T | The tracker must be able to withstand fast dumps of the solenoidal field without the development of excessive forces. Tracker module/detector design should mitigate the combined effect of a large field and pulsed currents that could weaken bond wires. |
| Other constraints from the machine |  | Focussing quads? Beam dump protection? Beampipe bakeout? |
| Tracker geometry |  |  |
| Inner Radius | Vertex Detector ~1.3 mm (should this be 13mm?) Tracker ~ 10-20 cm | Beam pipe expected to be 1.0-1.1 mm outer radius (should this be 10-11mm?) Normal assumption is separate vertex detector with an outer active layer at ~6 cm and some mechanics outside. |
| Outer Radius | 122 cm | Set by ECAL inner radius at 126 cm |
| Length | +/- 150 cm | Limited by ECAL |
| Angular Coverage | Typically to 10 deg (I suspect this is 10deg to the beam) | Includes longitudinal beam spot variation? |
| Performance |  |  |
| Momentum Resolution | 5×10-5/GeV | Higgs recoil mass from Z→µµ |
| Reconstruction momentum limit |  | What’s the lowest momentum for tracks we still want to be able to reconstruct? |
| Track reconstruction efficiency | Muons: close to 100% Charged pions: Electrons: in jets: |  |
| Charge misidentification |  |  |
| Pointing Resolution | Few mm? | Particle flow matching |
| Single-Hit Resolution | TBD | Depends on momentum resolution in 5T magnetic field |
| Two-Hit Separation | TBD | Need to find good metric |
| Timing | TBD |  |
| Readout/trigger requirements |  |  |
| Trigger rates | L0 L1 etc |  |
| Readout rate |  |  |
| Noise occupancy |  |  |
| SEU rates |  |  |
| Reconstruction requirements |  |  |
| Pattern Recognition | TBD | Need to find good metric, gg→hadron id? |
| Alignment | It must be possible to align the detector with tracks or an alignment system. It is required that the residual systematic on the momentum scale after alignment should be less than 5% of the intrinsic momentum scale. |  |
| Stability |  | Split into timescales - "seismic", days, run-to-run, within a run....? |
| Other requirements |  |  |
| Material Budget | Target 10-20% X0 total | Need to cross check with resolution and secondaries production |
| Power | TBD | Depends on cooling, which depends on material... |
| Monitoring/Instrumentation | Radiation monitoring |  |
|  | Acceleration monitoring | 0-250Hz |
| Integration | The tracker will be integrated and commissioned as one unit outside of the experiment |  |
| Access and Maintenance | Access to beam pipe Access to vertex Access to tracker Detector roll-in/roll-out |  |
| Thermal neutrality | The tracker (incl. services) must be thermally neutral across all of its interfaces to other sub-detectors |  |
| ESD protection |  |  |
| Grounding and shielding requirements |  |  |
| HV rating |  |  |
| Failure rates |  |  |
| Detector control system |  |  |
| Interlocks |  |  |