J. Grames (4/26/21)

I used Benno’s organization of the WBS structure to begin describing tasks & deliverables of the High Voltage Gun, Drive Laser, and Polarized Photocathodes

Title

Sources

Scoping statement

Design the electron and positron source systems, with prototypes of critical items

Deliverables

- Source system lattices

- Sources system design

- Sources performance specification

- Component requirements and specifications

- Component counts

- Design and costs for sources specific components

- Prototypes of critical items

- Chapter for EDR

Deliverables – High Voltage Gun

- Source system lattices

  - Is the 200 keV TDR scheme fully evaluated and optimized?

  - Evaluate injection for higher gun voltage, laser size (emittance), and pulse length

- Sources system design

  - Detail and describe major components of UHV inverted insulator geometry load lock gun (HV chamber, deposition chamber, loading chamber)

- Sources performance specification

  - No measurable field emission at operating voltage

  - Overvoltage of 100 kV required for processing

  - Achieve acceptable static and dynamic vacuum loads

- Component requirements and specifications

  - HV Design (PS, cable, vacuum insulator

  - HV cable/interface to ceramic

  - Electrostatic design (no field emission, cathode-anode optics, biased anode)

  - Vacuum design (modeling, NEG capacity, ion pumps)

- Component counts

  - HVPS, HV cable, insulators, vacuum chambers, manipulators, deposition materials, heaters, NEGs, ion pumps

- Design and costs for sources specific components

  - Many items are commercial

  - Engineering and design for HV cable and insulators

- Prototypes of critical items

  - 200 kV – recommend modifications to improve reliability and limit field emission

  - 200 to 300 kV – “a” + modeling and simulation , cathode-anode design

  - >300 kV – “b” + new commercial HV interface

- Chapter for EDR

  - Yes

Deliverables – Drive Laser

- Source system lattices

  - Solution is coupled to laser pulse length and size of injection scheme

- Sources system design

  - Is it worth re-evaluating GDE SLAC baseline?

  - Design fiber laser solution (master oscillator, amplifiers, OPA, helicity, optics)

- Sources performance specification

  - Laser wavelength

  - Frequency, phase jitter, pulse shape

  - Sufficient power, stability and pulse-pulse intensity noise

  - Fast and slow helicity reversal w/ suitable transition speeds and helicity pattern

  - Optical transport, laser size and position at photocathode, reflected light

- Component requirements and specifications

  - Master oscillator (seed, pre-amplifier, second harmonic generator)

  - Amplifier (pump, pre-amp, power-amp, second harmonic generator)

  - Optical power amplifier

  - Polarizers, Pockels cell, HV switch, helicity generator, quarter-wave plate

  - Lens, stages, remote controls, power meters, imaging software

  - Clean room, safety

- Component counts

- Design and costs for sources specific components

  - Most items are commercial, some require design at vendor

  - Engineering design for helicity reversal (HV switch, control boards)

- Prototypes of critical items

  - Yes, demonstrate essentials (master oscillator, amplifiers, OPA)

- Chapter for EDR

  - Yes

Deliverables – Polarized Photocathodes

- Source system lattices

  - Solution is coupled to peak charge density (pulse length and laser size)

- Sources system design

  - GaAs photocathode for polarization

  - Strained for high polarization

  - High surface p-doping (e.g. Zn) to mitigate surface charge limit

  - Relieve strain to minimize QE anisotropy

- Sources performance specification

  - Polarization >80% required, and >90% desired

  - QE >1% required and >5% desired

  - Achieve peak current density 14 A/cm2 (r=5mm) to 56 A/cm2 (r=2.5mm)

  - QE anisotropy < 6%

- Component requirements and specifications

  - Sufficient wafer size e.g. 2” or 3”

  - Uniformity of polarization and QE over laser area

  - Reliable performance from wafer to wafer

  - Arsenic capped

- Component counts

  - Reliable production supply chain

- Design and costs for sources specific components

  - Fabrication methodology (MBE, MOCVD, CBE)

  - Design of superlattice composition

  - Fabrication costs (deposition, materials, safety)

  - Testing and quality control

- Prototypes of critical items

  - Yes, high polarization photocathodes should be tested for polarization, QE and providing bunch charge/duty factor of ILC

- Chapter for EDR

  - Yes